Context Switching

- Register from R0 – R15 and xPSR values are considered as the context of a task. (17 registers)

- Out of these registers R0 – R3, R12, R14 are called caller saved registers, which means these registers are saved as part of the stacking operation. And these registers are called as a stack frame.

- When the currently running thread is interrupted by an exception handler for context switching. By default 7 registers from task context will be saved automatically before entering into the exception.

- In the above there are 8 (7 register needed for task context) registers, in LR register EXEC_RETURN instruction is stored

Context Switching Steps

    - Before entering into the exception handler above mentioned 7 register will be saved automatically

    - Now Control is entered into the exception handler so, the first step is to terminate the currently running task

    - For that we have store the remaining unsaved register from the context of a task.

    - So, inside the the exception handler we have to PUSH register from R4 – R11 on the stack. Because  R4-R11 are callee saved registers if the callee wants to alter these register it has to save these register before doing so.

    - Till now the stack pointer is pointing in the currently running task’s stack area.

    - Store the current stack pointer to currently running task’s task control block.

    - The currently running task’s context has been stored successfully

    - Now update the Stack pointer value of the processor with the next task’s stack pointer value.

    - If you look closely inside this exception handler, before terminating a task we are making all task’s current stack pointer to point to pushed R11 register, remember we pushed R4-R11

    - By doing POP from (R4-R11), these register values are restored (remember now it is pointing to the next task’s stack pointer)

    - As part of exception exit sequence, (R0 – R3), xPSR will be restored and using PC value the control will go the point where it left off in that task.

Starting the First Thread

- In the below image we have created a array of uint32_t for a stack of the task.

- If we point the stack pointer to address of R0 of the array at the end of the function, as part of the unstacking operation, it will unstack the register R0-R3 and the control will go the address mentioned in the PC.